// File: Area_Conservation.py

# -*- coding: utf-8 -*-

"""

计算颗粒分布的面积守恒性，批量处理ZDEM模拟的.dat文件，

生成并分析指定颜色颗粒的三角网格，输出三角网格图及面积统计。

"""

import os

import sys

import getopt

import re

import numpy as np

import matplotlib

matplotlib.use('Agg')

import matplotlib.pyplot as plt

from matplotlib.tri import Triangulation

import matplotlib.ticker as mticker

import warnings

from scipy.spatial import Delaunay

try:

sys.stdout.reconfigure(encoding='utf-8')

except Exception:

import io

sys.stdout = io.TextIOWrapper(sys.stdout.buffer, encoding='utf-8')

sys.path.append(os.path.dirname(os.path.abspath(__file__)))

import zdemio

import zdemplot

# 全局字体与字号

plt.rcParams.update({

'font.family': ['Times New Roman', 'Microsoft YaHei', 'SimHei'],

'font.size': 10,

'axes.labelsize': 10,

'axes.titlesize': 10,

'xtick.labelsize': 9,

'ytick.labelsize': 9,

'legend.fontsize': 9,

'axes.unicode_minus': False,

'savefig.bbox': 'tight',

'savefig.pad_inches': 0.05,

'pdf.fonttype': 42,

'ps.fonttype': 42,

'svg.fonttype': 'none',

'axes.linewidth': 1.0,

'xtick.major.width': 0.8,

'ytick.major.width': 0.8,

'xtick.minor.width': 0.6,

'ytick.minor.width': 0.6,

})

# 常量 (命令行参数可覆盖)

COLOR_TO_EXTRACT = None

THRESHOLD_FACTOR = 3.0        # 三角网格边长阈值因子

DEFAULT_DATA_DIR = "data"       # 默认数据目录

PLOT_ORIGINAL_TRI = True       # 是否绘制原始三角网格

PLOT_FILTERED_TRI = True       # 是否绘制过滤后三角网格

PLOT_AREA_TREND = 'raw'         # 面积趋势图类型: 'raw', 'percentage', 'normalized'

def usage(software_name):

print(f"用法: python {software_name} [选项]")

print("  --dir=<目录>           指定包含 .dat 文件的目录 (默认: data)")

print("  --colors=<颜色编号,...>  指定要提取的颗粒颜色编号 (逗号分隔, 默认: all，支持 all 或 * 表示全部颜色)")

print("  --threshold=<浮点数>     指定边长阈值因子 (默认: 3.0)")

print("  --threshold-percent=<浮点数> 面积趋势图阈值百分比，控制绿色安全带与红/绿阈值线位置 (默认: 5.0)")

print("  --hint-band-percent=<浮点数> 提示区（绿色带）宽度百分比，默认与阈值一致")

print("  --plot-original-tri=<true|false> 是否绘制原始三角网格图 (默认: true)")

print("  --plot-filtered-tri=<true|false> 是否绘制过滤后三角网格图 (默认: true)")

print("  --plot-area-type=<raw|percentage|normalized> 面积趋势图类型 (默认: raw)")

print("  --paper-style=<true|false> 应用论文风格（更粗的坐标轴、统一字号等） (默认: false)")

print("  --xmax=<浮点数>         指定实验区域的X轴最大值")

print("  --ymax=<浮点数>         指定实验区域的Y轴最大值")

print("  -h                     显示此帮助信息")

def read_surface_particles(filename):

"""从_particles.txt文件中读取颗粒信息"""

particles_data = []

try:

with open(filename, 'r', encoding='utf-8') as f:

lines = f.readlines()

if len(lines) < 4:

warnings.warn(f"文件 '{filename}' 内容不足，无法解析颗粒数据。", UserWarning)

return None, None, None

for line in lines[3:]:

if line.strip().startswith('-'):

break

parts = line.strip().split('\t')

if len(parts) >= 5:

try:

x = float(parts[1])

y = float(parts[2])

radius = float(parts[3])

color = int(float(parts[4]))

particles_data.append((x, y, radius, color))

except ValueError:

warnings.warn(f"跳过文件 '{filename}' 中格式错误的行: {line.strip()}", UserWarning)

continue

if not particles_data:

warnings.warn(f"在 '{filename}' 中没有找到有效的颗粒数据。", UserWarning)

return None, None, None

coords = np.array([[p[0], p[1]] for p in particles_data], dtype=np.float32)

radii = np.array([[p[2]] for p in particles_data], dtype=np.float32)

colors = np.array([[p[3]] for p in particles_data], dtype=np.int32)

return coords, radii, colors

except FileNotFoundError:

print(f"错误: 文件 '{filename}' 未找到。")

return None, None, None

except Exception as e:

print(f"读取文件 '{filename}' 时发生未知错误: {e}")

return None, None, None

def create_triangulation(coords):

"""创建三角网格划分"""

if coords is None or coords.shape[0] < 3:

print("颗粒数量不足，无法创建三角网格 (至少需要3个颗粒)。")

return None

if coords.shape[0] > 2:

unique_coords = np.unique(coords, axis=0)

if unique_coords.shape[0] < 3:

warnings.warn("警告: 颗粒点存在大量重复或几乎共线，三角剖分可能不准确或失败。", UserWarning)

return None

try:

delaunay = Delaunay(coords)

tri = Triangulation(coords[:, 0], coords[:, 1], triangles=delaunay.simplices)

if len(tri.triangles) == 0:

warnings.warn("警告: 没有生成任何三角形。", UserWarning)

return None

return tri

except Exception as e:

print(f"创建三角网格时出错: {e}")

return None

def filter_triangles(tri, coords, radii, threshold_factor):

"""过滤边长大于阈值的三角形。"""

if tri is None or coords is None or radii is None or len(radii) == 0:

warnings.warn("无效的输入数据，无法过滤三角形。", UserWarning)

return None

triangles = tri.triangles

avg_radius = np.mean(radii)

threshold = threshold_factor * avg_radius

triangle_coords = coords[triangles]

p1 = triangle_coords[:, 0, :]

p2 = triangle_coords[:, 1, :]

p3 = triangle_coords[:, 2, :]

edge1 = np.linalg.norm(p1 - p2, axis=1)

edge2 = np.linalg.norm(p2 - p3, axis=1)

edge3 = np.linalg.norm(p3 - p1, axis=1)

mask = (edge1 <= threshold) & (edge2 <= threshold) & (edge3 <= threshold)

filtered_triangles = triangles[mask]

if len(filtered_triangles) > 0:

new_tri = Triangulation(tri.x, tri.y, filtered_triangles)

return new_tri

else:

warnings.warn(f"警告: 过滤后没有剩余三角形 (阈值 = {threshold:.2f})。", UserWarning)

return None

def calculate_triangle_areas(tri, coords):

"""计算三角形面积并求和"""

if tri is None or coords is None or tri.triangles.shape[0] == 0:

return 0.0

triangles = tri.triangles

p1 = coords[triangles[:, 0]]

p2 = coords[triangles[:, 1]]

p3 = coords[triangles[:, 2]]

vec1 = p2 - p1

vec2 = p3 - p1

cross_product_z = vec1[:, 0] * vec2[:, 1] - vec1[:, 1] * vec2[:, 0]

areas = 0.5 * np.abs(cross_product_z)

total_area = np.sum(areas)

return float(total_area)

def export_particles_txt(coords, radii, colors, original_dat_file_path):

"""根据原始 dat 文件名，在 data 目录下生成对应的 _particles.txt 文件"""

if coords is None or radii is None or colors is None or coords.shape[0] == 0:

warnings.warn(f"没有有效的颗粒数据可供导出到文本文件。", UserWarning)

return

if not original_dat_file_path:

warnings.warn(f"未提供原始 .dat 文件路径，无法确定输出文件名。", UserWarning)

return

data_dir = os.path.join(os.path.dirname(os.path.abspath(__file__)), DEFAULT_DATA_DIR)

os.makedirs(data_dir, exist_ok=True)

base_name = os.path.splitext(os.path.basename(original_dat_file_path))[0]

output_file = os.path.join(data_dir, f'{base_name}_particles.txt')

try:

with open(output_file, 'w', encoding='utf-8') as f:

f.write("表面颗粒信息:\n")

f.write("序号\tX坐标\tY坐标\t半径\t颜色编号\n")

f.write("-" * 50 + "\n")

for i in range(coords.shape[0]):

x = coords[i, 0]

y = coords[i, 1]

radius = radii[i, 0]

color = int(colors[i, 0])

f.write(f"{i}\t{x:.2f}\t{y:.2f}\t{radius:.2f}\t{color}\n")

f.write("-" * 50 + "\n")

f.write(f"表面颗粒总数: {coords.shape[0]}\n")

print(f"颗粒信息已保存到 '{output_file}'。")

except Exception as e:

print(f"写入文件 '{output_file}' 失败: {e}")

def dat_to_particles_txt_batch(data_dir):

"""批量将指定 data 目录下的所有 .dat 文件导出为 _particles.txt"""

data_dir_abs = os.path.join(os.path.dirname(os.path.abspath(__file__)), data_dir)

if not os.path.exists(data_dir_abs):

warnings.warn(f"目录不存在: '{data_dir_abs}'。跳过 .dat 文件转换。", UserWarning)

return

dat_files = [f for f in os.listdir(data_dir_abs) if f.endswith('.dat')]

if not dat_files:

print(f"在 '{data_dir_abs}' 中没有找到任何 .dat 文件。")

return

for file_name in sorted(dat_files):

dat_file_path = os.path.join(data_dir_abs, file_name)

try:

result = zdemio.read_data(dat_file_path)

if result is None:

warnings.warn(f"'{file_name}' 解析失败，zdemio.read_data 返回 None。", UserWarning)

continue

if len(result) == 5:

_WALL, BALL, _CONTACT, _BOND, _CurrentStep = result

elif len(result) == 6:

_WALL, BALL, _CONTACT, _BOND, _CurrentStep, _ = result

else:

warnings.warn(f"'{file_name}' 解析返回值数量异常: {len(result)}。跳过。", UserWarning)

continue

_, BALLxyN2_raw, BALLRadN1_raw, BALLColorN1_raw = zdemio.BallListStrToNumpyArray(BALL)

BALLColorN1_reshaped = BALLColorN1_raw.reshape(-1, 1)

export_particles_txt(BALLxyN2_raw, BALLRadN1_raw, BALLColorN1_reshaped, dat_file_path)

except Exception as e:

print(f"处理 '{file_name}' 时出错: {e}")

def process_multiple_files(data_dir, color_list, threshold_factor):

"""处理data目录下的所有_particles.txt文件，进行分析和绘图"""

results = []

data_dir_abs = os.path.join(os.path.dirname(os.path.abspath(__file__)), data_dir)

if not os.path.exists(data_dir_abs):

print(f"错误: 目录 '{data_dir_abs}' 不存在。")

return results

particle_files = [f for f in os.listdir(data_dir_abs) if f.endswith('_particles.txt')]

if not particle_files:

warnings.warn(f"在 '{data_dir_abs}' 中没有找到任何 _particles.txt 文件。", UserWarning)

return results

particle_files.sort(key=lambda f: int(re.findall(r'\d+', f)[-1]) if re.findall(r'\d+', f) else 0)

for file_name in particle_files:

file_path = os.path.join(data_dir_abs, file_name)

coords, radii, colors = read_surface_particles(file_path)

if coords is None or radii is None or colors is None:

warnings.warn(f"文件 '{file_name}' 缺少有效的颗粒数据，跳过该文件。", UserWarning)

continue

# 如果未指定颜色列表（None），默认选择全部颗粒

if color_list is None:

color_indices = list(range(colors.shape[0]))

else:

color_indices = [i for i in range(colors.shape[0]) if colors[i, 0] in color_list]

if not color_indices:

continue

filtered_coords = coords[color_indices]

filtered_radii = radii[color_indices]

filtered_colors = colors[color_indices]

if filtered_coords.shape[0] < 3:

warnings.warn(f"筛选后的颗粒数量不足3个 ({filtered_coords.shape[0]}个)，无法进行三角剖分，跳过文件 '{file_name}'。", UserWarning)

continue

original_tri = create_triangulation(filtered_coords)

if original_tri is not None:

filtered_tri = filter_triangles(original_tri, filtered_coords, filtered_radii, threshold_factor)

if filtered_tri is not None:

total_area = calculate_triangle_areas(filtered_tri, filtered_coords)

results.append({

'file_name': file_name,

'total_particles': filtered_coords.shape[0],

'area': total_area,

'coords': filtered_coords,

'radii': filtered_radii,

'colors': filtered_colors,

'original_tri': original_tri,

'tri': filtered_tri

})

return results

def compare_results(results):

"""比较不同文件的颗粒数量和面积结果"""

if not results:

print("没有可比较的结果。")

return

print("\n=== 结果比较 ===")

print("文件名\t\t颗粒数量\t面积(平方单位)")

print("-" * 50)

sorted_results = sorted(results, key=lambda x: int(re.findall(r'\d+', x['file_name'])[-1]) if re.findall(r'\d+', x['file_name']) else 0)

for result in sorted_results:

rounded_area_display = round(result['area'], -3) if result['area'] >= 1000 else result['area']

print(f"{result['file_name']}\t{result['total_particles']}\t{rounded_area_display:.0f}")

print("\n=== 相邻文件变化分析 ===")

for i in range(1, len(sorted_results)):

prev_result = sorted_results[i-1]

curr_result = sorted_results[i]

prev_area = prev_result['area']

curr_area = curr_result['area']

area_change = ((curr_area - prev_area) / prev_area) * 100 if prev_area != 0 else float('inf')

prev_particles = prev_result['total_particles']

curr_particles = curr_result['total_particles']

particle_change = ((curr_particles - prev_particles) / prev_particles) * 100 if prev_particles != 0 else float('inf')

print(f"\n从 '{prev_result['file_name']}' 到 '{curr_result['file_name']}':")

print(f"面积变化率: {area_change:+.2f}%")

print(f"颗粒数量变化率: {particle_change:+.2f}%")

if len(sorted_results) >= 2:

first_area = sorted_results[0]['area']

last_area = sorted_results[-1]['area']

area_change_overall = ((last_area - first_area) / first_area) * 100 if first_area != 0 else float('inf')

first_particles = sorted_results[0]['total_particles']

last_particles = sorted_results[-1]['total_particles']

particle_change_overall = ((last_particles - first_particles) / first_particles) * 100 if first_particles != 0 else float('inf')

print("\n=== 总体变化分析 ===")

print(f"从 '{sorted_results[0]['file_name']}' 到 '{sorted_results[-1]['file_name']}'")

print(f"面积变化率: {area_change_overall:+.2f}%")

print(f"颗粒数量变化率: {particle_change_overall:+.2f}%")

def save_triangulation_plot(coords, radii, colors, color_list, tri, filename, filtered=False, xmax=70000.0, ymax=25000.0):

"""保存三角网格图（叠加颗粒），输出 PDF + SVG。"""

if coords is None or coords.shape[0] == 0:

warnings.warn(f"没有坐标数据，无法绘制三角网格图到 '{filename}'。", UserWarning)

return

if colors is not None:

# 全部映射为0，保证所有颗粒都用同一种颜色

colors = np.zeros_like(colors)

if len(colors.shape) == 1:

colors = colors.reshape(-1, 1)

colors = np.array([c.reshape(1, 1) for c in colors])

# 只用一种颜色（绿色）

color_list = [(0, 1, 0)]

fig, ax = plt.subplots(figsize=(6.69, 3.94), facecolor='white', dpi=300)

ax.set_facecolor('white')

ax.set_xlim(0, xmax)

ax.set_ylim(0, ymax)

if coords is not None and radii is not None and colors is not None:

# 过滤超出xmax和ymax范围的颗粒

mask = (coords[:, 0] <= xmax) & (coords[:, 1] <= ymax)

filtered_coords = coords[mask]

filtered_radii = radii[mask]

filtered_colors = colors[mask]

if len(filtered_coords) > 0:

zdemplot.plot_ball(fig, ax, filtered_coords, filtered_radii, filtered_colors, color_list)

else:

warnings.warn(f"没有颗粒在指定区域内 (xmax={xmax}, ymax={ymax})，跳过绘制颗粒到 '{filename}'。", UserWarning)

else:

warnings.warn(f"没有有效的颗粒数据，跳过绘制颗粒到 '{filename}'。", UserWarning)

if tri is not None and tri.triangles.shape[0] > 0:

ax.triplot(tri.x, tri.y, tri.triangles, color='lime', linestyle='-', alpha=1.0, linewidth=0.2)

else:

warnings.warn(f"没有有效的三角网格数据，跳过绘制网格线到 '{filename}'。", UserWarning)

ax.set_xlim(0, xmax)

ax.set_ylim(0, ymax)

zdemplot.zdem_fig_set(

fig, ax,

xmaxdefine='true', ymaxdefine='true', xmindefine='true', ymindefine='true',

xmin=0.0, xmax=xmax, ymin=0.0, ymax=ymax,

wbleft=0.0, wbright=xmax, wbbottom=0.0, wbtop=ymax,

leftshow='true', rightshow='true', bottomshow='true', topshow='true',

major_locator=10000.0, minor_locator=1000.0,

fontsize=8, linewidth=0.5, pagesize=6.69

)

base = os.path.splitext(filename)[0]

pdf_path = base + '.pdf'

svg_path = base + '.svg'

fig.savefig(pdf_path, format='pdf', bbox_inches='tight', pad_inches=0.05)

fig.savefig(svg_path, format='svg', bbox_inches='tight', pad_inches=0.05)

plt.close(fig)

print(f"{'过滤后' if filtered else '原始'}三角网格图已保存到 '{pdf_path}' 与 '{svg_path}'。")

def plot_area_trend(results, plot_type='percentage', y_axis_margin=1.0, threshold_percent=5.0, paper_style=False, hint_band_percent=None):

"""绘制面积趋势曲线，输出 PDF + SVG。"""

if not results:

print("没有可用于绘制面积趋势图的结果。")

return

sorted_results = sorted(results, key=lambda x: int(re.findall(r'\d+', x['file_name'])[-1]) if re.findall(r'\d+', x['file_name']) else 0)

file_numbers = [int(re.findall(r'\d+', r['file_name'])[-1]) for r in sorted_results if re.findall(r'\d+', r['file_name'])]

areas = [r['area'] for r in sorted_results if re.findall(r'\d+', r['file_name'])]

if not file_numbers or not areas:

warnings.warn("未能从结果中提取有效的绘图数据，无法绘制面积趋势图。", UserWarning)

return

# X轴归一化

if len(file_numbers) > 1:

start_step = file_numbers[0]

end_step = file_numbers[-1]

step_range = end_step - start_step

if step_range == 0:

x_data = [0.0 for _ in file_numbers]

else:

x_data = [(f - start_step) / step_range for f in file_numbers]

else:

x_data = [0.0]

base_area = float(areas[0])

if hint_band_percent is None:

hint_band_percent = threshold_percent

if base_area != 0:

y_data_percent = [((a - base_area) / base_area) * 100.0 for a in areas]

else:

y_data_percent = [0.0 for _ in areas]

if plot_type == 'raw':

y_data = areas

y_label = '面积 (平方单位)'

elif plot_type == 'normalized':

min_a, max_a = min(areas), max(areas)

if max_a != min_a:

y_data = [(a - min_a)/(max_a - min_a) for a in areas]

else:

y_data = [0.5 for _ in areas]

y_label = '归一化面积 (0-1)'

else:

y_data = y_data_percent

y_label = '面积相对变化 (%)'

fig, ax = plt.subplots(figsize=(6.69, 3.94), facecolor='white', dpi=300)

if paper_style:

for spine in ax.spines.values():

spine.set_linewidth(1.2)

ax.tick_params(axis='both', which='both', width=1.0, labelsize=9)

ax.plot(x_data, y_data, marker='o', markersize=4, linestyle='-', color='#1f77b4', linewidth=1.5, label='面积波动', alpha=0.9)

ax.set_xlabel('归一化时间步 (0 → 1)', fontsize=10 if paper_style else None)

ax.set_ylabel(y_label, fontsize=10 if paper_style else None)

ax.grid(True, which='both', linestyle='--', linewidth=0.5, alpha=0.5)

ax.set_xlim(0, 1.05)

if plot_type == 'percentage' or plot_type == 'default':

ax.axhline(0, color='gray', linestyle='-', linewidth=0.8, alpha=0.5)

ax.axhline(threshold_percent, color='red', linestyle='--', linewidth=1.0, label=f'±{threshold_percent}% 阈值', alpha=0.7)

ax.axhline(-threshold_percent, color='red', linestyle='--', linewidth=1.0, alpha=0.7)

ax.axhspan(-hint_band_percent, hint_band_percent, facecolor='green', alpha=0.08, label='安全波动区间')

max_fluctuation = max(max([abs(y) for y in y_data]), threshold_percent)

y_limit = max_fluctuation * 2.5

y_limit = max(y_limit, 10.0)

ax.set_ylim(-y_limit, y_limit)

ax.yaxis.set_major_formatter(mticker.PercentFormatter(xmax=100, decimals=1))

elif plot_type == 'raw':

margin = (max(y_data) - min(y_data)) * 0.5

if margin == 0: margin = 1.0

ax.set_ylim(min(y_data) - margin, max(y_data) + margin)

ax.legend(loc='upper right', fontsize=9, frameon=True, fancybox=False, edgecolor='black')

try:

data_dir = os.path.join(os.path.dirname(os.path.abspath(__file__)), 'data')

except NameError:

data_dir = 'data'

os.makedirs(data_dir, exist_ok=True)

base_path = os.path.join(data_dir, f'area_trend')

pdf_path = base_path + '.pdf'

svg_path = base_path + '.svg'

fig.savefig(pdf_path, format='pdf', bbox_inches='tight', pad_inches=0.1)

fig.savefig(svg_path, format='svg', bbox_inches='tight', pad_inches=0.1)

plt.close(fig)

print(f"面积趋势图（{plot_type}）已保存到: {pdf_path} 与 {svg_path}")

def main():

data_dir = DEFAULT_DATA_DIR

color_list_to_extract = COLOR_TO_EXTRACT

threshold_factor = THRESHOLD_FACTOR

threshold_percent = 5.0

y_axis_margin = 1.0

hint_band_percent = None

plot_original_tri_flag = PLOT_ORIGINAL_TRI

plot_filtered_tri_flag = PLOT_FILTERED_TRI

plot_area_type = PLOT_AREA_TREND

paper_style_flag = False

xmax = 70000.0  # 默认值

ymax = 25000.0  # 默认值

try:

opts, args = getopt.getopt(sys.argv[1:], "h",\

longopts=['dir=','colors=','threshold=','threshold-percent=','hint-band-percent=','y-margin=','plot-original-tri=','plot-filtered-tri=', 'plot-area-type=', 'paper-style=', 'xmax=', 'ymax='])

except getopt.GetoptError as err:

print(f"命令行参数错误: {err}")

usage(sys.argv[0])

sys.exit(2)

for op, value in opts:

if op == "-h":

usage(sys.argv[0])

sys.exit()

elif op == "--dir":

data_dir = value

elif op == "--colors":

v = value.strip().lower()

if v in ["all", "*", ""]:

color_list_to_extract = None  # 选择全部颜色

else:

try:

color_list_to_extract = [int(c.strip()) for c in value.split(',')]

except ValueError:

print("错误: --colors 参数需要整数列表，例如: --colors=7,8，或使用 all/* 表示全部颜色。")

usage(sys.argv[0])

sys.exit(2)

elif op == "--threshold":

try:

threshold_factor = float(value)

except ValueError:

print("错误: --threshold 参数需要浮点数，例如: --threshold=3.5。")

usage(sys.argv[0])

sys.exit(2)

elif op == "--threshold-percent":

try:

threshold_percent = float(value)

except ValueError:

print("错误: --threshold-percent 参数需要浮点数，例如: --threshold-percent=4.0。")

usage(sys.argv[0])

sys.exit(2)

elif op == "--hint-band-percent":

try:

hint_band_percent = float(value)

except ValueError:

print("错误: --hint-band-percent 参数需要浮点数，例如: --hint-band-percent=3.0。")

usage(sys.argv[0])

sys.exit(2)

elif op == "--y-margin":

try:

y_axis_margin = float(value)

except ValueError:

print("错误: --y-margin 参数需要浮点数，例如: --y-margin=1.0。")

usage(sys.argv[0])

sys.exit(2)

elif op == "--plot-original-tri":

plot_original_tri_flag = value.lower() == 'true'

elif op == "--plot-filtered-tri":

plot_filtered_tri_flag = value.lower() == 'true'

elif op == "--plot-area-type":

if value.lower() in ['raw', 'percentage', 'normalized']:

plot_area_type = value.lower()

else:

print("错误: --plot-area-type 参数只能是 'raw', 'percentage' 或 'normalized'。")

usage(sys.argv[0])

sys.exit(2)

elif op == "--paper-style":

paper_style_flag = value.lower() == 'true'

elif op == "--xmax":

try:

xmax = float(value)

except ValueError:

print("错误: --xmax 参数需要浮点数，例如: --xmax=9.0。")

usage(sys.argv[0])

sys.exit(2)

elif op == "--ymax":

try:

ymax = float(value)

except ValueError:

print("错误: --ymax 参数需要浮点数，例如: --ymax=15.0。")

usage(sys.argv[0])

sys.exit(2)

print("--- 开始处理ZDEM颗粒分布与面积守恒分析 ---")

print(f"数据目录: {data_dir}")

print(f"提取颜色: {'全部颜色' if color_list_to_extract is None else color_list_to_extract}")

print(f"阈值因子: {threshold_factor}")

dat_to_particles_txt_batch(data_dir)

results = process_multiple_files(data_dir=data_dir, color_list=color_list_to_extract, threshold_factor=threshold_factor)

compare_results(results)

plot_area_trend(results, plot_type=plot_area_type, y_axis_margin=y_axis_margin, threshold_percent=threshold_percent, paper_style=paper_style_flag, hint_band_percent=hint_band_percent)

data_dir_abs = os.path.join(os.path.dirname(os.path.abspath(__file__)), data_dir)

triangulation_dir = os.path.join(data_dir_abs, '1_triangulation_plot')

filtered_triangulation_dir = os.path.join(data_dir_abs, '2_filtered_triangulation_plot')

if not results:

warnings.warn("没有可用于绘图的结果，跳过图片保存。", UserWarning)

return

for result in results:

file_name = result.get('file_name')

coords = result.get('coords')

radii = result.get('radii')

colors = result.get('colors')

original_tri = result.get('original_tri')

filtered_tri = result.get('tri')

if colors is not None:

colors = np.array(colors)

if len(colors.shape) == 1:

colors = colors.reshape(-1, 1)

colors = np.array([c.reshape(1, 1) for c in colors])

if file_name is None or coords is None or radii is None or colors is None:

warnings.warn(f"跳过文件 '{file_name}' 的绘图，因为数据不完整。", UserWarning)

continue

base_name = os.path.splitext(file_name)[0]

print(f"\n正在绘制并保存 '{file_name}' 的图形...")

if plot_original_tri_flag:

os.makedirs(triangulation_dir, exist_ok=True)

tri_img = os.path.join(triangulation_dir, f'{base_name}_triangulation.jpg')

save_triangulation_plot(coords, radii, colors, None, original_tri, tri_img, filtered=False, xmax=xmax, ymax=ymax)

if plot_filtered_tri_flag:

os.makedirs(filtered_triangulation_dir, exist_ok=True)

filtered_img = os.path.join(filtered_triangulation_dir, f'{base_name}_filtered_triangulation.jpg')

save_triangulation_plot(coords, radii, colors, None, filtered_tri, filtered_img, filtered=True, xmax=xmax, ymax=ymax)

print("--- ZDEM颗粒分布与面积守恒分析完成 ---")

if __name__ == "__main__":

main()

// File: main.py

#! /usr/bin/env python3

# -*- coding: utf-8 -*-

"""

读取 VBOX 计算结果，批量生成颗粒与 bond 的 jpg 图片，支持并行绘图。

"""

import logging

from concurrent.futures import ProcessPoolExecutor

import os, sys, getopt, time

import os.path #判断文件是否存在

import matplotlib.ticker as ticker

#import multiprocessing

import zdemio

import zdemplot

import matplotlib as mpl

mpl.use('Agg')

import matplotlib.pyplot as plt

import checkLicense

import io

sys.stdout = io.TextIOWrapper(sys.stdout.buffer,encoding='utf-8')

logger = logging.getLogger('test_logger')

logger.setLevel(logging.WARNING)  #设置默认的日志级别 CRITICAL > ERROR > WARNING > INFO > DEBUG

#创建StreamHandler对象

sh = logging.StreamHandler()

#StreamHandler对象自定义日志级别

sh.setLevel(logging.INFO)

#StreamHandler对象自定义日志格式

#sh.setFormatter(formatter)

sh.flush()

logger.addHandler(sh)  #logger日志对象加载StreamHandler对象

softwareName=sys.argv[0]

version = '2.2'

VBOXscriptDir=sys.path[0]

sys.path.append(VBOXscriptDir)

total_list = []

#print ("VBOXscriptDir",VBOXscriptDir)

#print ("参数个数：",len(sys.argv))

#reg=checkLicense.register(version,VBOXscriptDir)

#reg.checkAuthored()

#print ('参数个数为:', len(sys.argv))

#print ('参数列表:', str(sys.argv))

if (len(sys.argv) < 2):

zdemio.usage(softwareName)

sys.exit()

opts, args = getopt.getopt(sys.argv[1:], "hv",\

longopts=['dir=','xmax=','ymax=','xmove=','ymove=','xmin=','ymin=',\

'major_locator=','minor_locator=','fontsize=','max_workers=','dpi=',\

'linewidth=','pagesize=','leftshow=','rightshow=','bottomshow=','topshow=',\

'wallshow=','colormap=','surfaceshow=','bondplot=','ballplot='])

input_file=""

output_file=""

DataDir=""

xmax=1000000000.0

ymax=1000000000.0

xmaxdefine='false'

ymaxdefine='false'

xmove=0.0

ymove=0.0

xmin=0.0

ymin=0.0

xmindefine='false'

ymindefine='false'

major_locator=10000.0

minor_locator=1000.0

fontsize=9

max_workers = 4

#print("max_workers:%d" %(max_workers) )

dpi=600

linewidth=0.5

pagesize=14

topshow='true'

rightshow='true'

bottomshow='true'

leftshow='true'

wallshow='true'

surfaceshow= 'false'

colormap='default'

bondplot='false'

ballplot='true'

for op, value in opts:

if op == "-h":

zdemio.usage(softwareName)

sys.exit()

elif op == "-v":

print ("%s" %(version))

sys.exit()

elif op == "--dir":

DataDir = value

print ("DataDir", DataDir)

elif op == "--xmax":

xmax = float(value)

xmaxdefine='true'

elif op == "--ymax":

ymax = float(value)

ymaxdefine='true'

elif op == "--xmove":

xmove = float(value)

elif op == "--ymove":

ymove = float(value)

elif op == "--xmin":

xmin = float(value)

xmindefine='true'

elif op == "--ymin":

ymin = float(value)

ymindefine='true'

elif op == "--major_locator":

major_locator = float(value)

elif op == "--minor_locator":

minor_locator = float(value)

elif op == "--fontsize":

fontsize = float(value)

elif op == "--max_workers":

max_workers = int(value)

elif op == "--dpi":

dpi = int(value)

elif op == "--linewidth":

linewidth = float(value)

elif op == "--pagesize":

pagesize = float(value)

elif op == "--topshow":

topshow=value

elif op == "--rightshow":

rightshow=value

elif op == "--bottomshow":

bottomshow=value

elif op == "--leftshow":

leftshow=value

elif op == "--wallshow":

wallshow=value

elif op == "--surfaceshow":

surfaceshow=value

elif op == "--colormap":

colormap=value

elif op == "--bondplot":

bondplot=value

elif op == "--ballplot":

ballplot=value

if len(sys.argv) < 2:

usage()

os._exit(0)

#颜色查询

colormapfile= zdemio.GetColormapFile(colormap,VBOXscriptDir)

logger.info("colormapfile:%s" %(colormapfile) )

flist = []

plt.close('all')

ColorList, ColorMap = zdemplot.get_color_map(colormapfile)

#get <*.dat> file

VBOXfile= zdemio.get_file_list(DataDir, FileNamePrefix='all_', FileNameSuffix='.dat')

def compute_polygon_area(points):

point_num = len(points)

if(point_num < 3): return 0.0

s = points[0][1] * (points[point_num-1][0] - points[1][0])

for i in range(1, point_num):

s += points[i][1] * (points[i-1][0] - points[(i+1)%point_num][0])

return abs(s/2.0)

def read_and_gen_fig(file):

print("read file:%s"%(file),flush=True)

WALL,BALL,CONTACT,BOND,CurrentStep, Group=zdemio.read_data(file)

test = []

lef_wall = []

rig_wall = []

top_wall_left_x = float(WALL[7][4])

top_wall_left_y = float(WALL[7][5])

top_wall_right_x = float(WALL[7][2])

top_wall_right_y = float(WALL[7][3])

top_wall_left_y1 = float(WALL[8][5])

bottom_wall_left_x = float(WALL[4][2])

bottom_wall_left_y = float(WALL[4][3])

bottom_wall_left_x1 = float(WALL[5][2])

bottom_wall_left_y1 = float(WALL[5][3])

bottom_wall_right_x = float(WALL[4][4])

bottom_wall_right_y = float(WALL[4][5])

bottom_wall_right_x1 = float(WALL[6][4])

bottom_wall_right_y1 = float(WALL[6][5])

area1 = (bottom_wall_right_x - bottom_wall_left_x1) * (bottom_wall_left_y - bottom_wall_left_y1)

area2 = (top_wall_right_x - top_wall_left_x) * (top_wall_left_y1 - top_wall_left_y)

total_area = area1 + area2

total_list.append(total_area)

# with open(r'E:\scientific_research\project\test\data\area_subtract.txt', 'a') as file:

# 	file.write(str(total_area))

# 	file.write('\n')

for i in range(len(Group)):

if Group[i][1] == 'lef_wall':

for j in range(len(BALL)):

if BALL[j][1] == Group[i][0]:

test.append(BALL[j])

lef_wall.append(BALL[j])

if Group[i][1] == 'rig_wall':

for j in range(len(BALL)):

if BALL[j][1] == Group[i][0]:

test.append(BALL[j])

rig_wall.append(BALL[j])

BALL = test

Ball_position = []

# Ball_position.append([float(bottom_wall_left_x), float(bottom_wall_left_y)])

for k in range(len(lef_wall)):

Ball_position.append([float(lef_wall[k][2]), float(lef_wall[k][3])])

# Ball_position.append([float(top_wall_left_x), float(top_wall_left_y)])

# Ball_position.append([float(top_wall_right_x), float(top_wall_right_y)])

for k in range(len(rig_wall)):

Ball_position.append([float(rig_wall[k][2]), float(rig_wall[len(rig_wall) - 1 - k][3])])

# Ball_position.append([float(bottom_wall_right_x), float(bottom_wall_right_y)])

filename = file.split('\\')[5].split('.')[0]

with open(r'E:\scientific_research\project\test\data\{}.txt'.format(filename), 'a') as file:

for i in range(len(Ball_position)):

file.write(str(Ball_position[i][0]) + ',' +str(Ball_position[i][1]))

file.write('\n')

# print(Ball_position)

#

#print(compute_polygon_area(Ball_position))

#List to array

BALLIdN1, BALLxyN2, BALLRadN1, BALLColor = zdemio.BallListStrToNumpyArray(BALL)

WALLIdN1, WALLP1P2xyxyN4 = zdemio.WallListStrToNumpyArray(WALL)

WALLP1P2xyxyN4, BALLxyN2 = zdemio.xy_move(WALLP1P2xyxyN4, BALLxyN2, xmove, ymove)

wbleft,wbright,wbbottom,wbtop = zdemplot.search_domain(WALLP1P2xyxyN4, BALLxyN2, BALLRadN1)

#print(wbleft,wbright,wbbottom,wbtop )

#plot ball

if ballplot =='true':

logger.info("plot ball...")

tic = time.time()

figball=plt.figure(1)

#	ax = subplot(111,aspect='equal')

axball=plt.gca()

#bleft,bright,bbottom,btop = zdemplot.plot_ball(fig,ax,Ball,ColorList)

zdemplot.plot_ball(figball,axball,BALLxyN2, BALLRadN1, BALLColor, ColorList)

#print(wbleft,wbright,wbbottom,wbtop )

#print('wallshow',wallshow)

if (wallshow == 'true'):

zdemplot.plot_wall(figball,axball,WALLP1P2xyxyN4,ColorList,linewidth=linewidth)

if (surfaceshow == 'true'):

zdemplot.plot_surface(figball,axball,BALLxyN2, BALLRadN1,linewidth=linewidth)

#set figure

zdemplot.zdem_fig_set(figball,axball,xmaxdefine, ymaxdefine, xmindefine, ymindefine,

xmin,xmax,ymin,ymax,

wbleft,wbright,wbbottom,wbtop,

leftshow,rightshow,bottomshow,topshow,

major_locator,minor_locator,fontsize,linewidth,pagesize)

VBOXfileJPG = file.replace('.dat','.jpg')

figball.savefig(VBOXfileJPG,dpi=dpi, bbox_inches="tight")#, pad_inches=0.0)

#fig.savefig(figName+'.pdf',dpi=300, bbox_inches="tight")

plt.close(figball)

toc = time.time()

logger.info('used {:.5}s'.format(toc-tic))

#plot bond

if bondplot =='true':

logger.info("plot bond...")

CONTACTId1Id2N2,BONDId1Id2N2,BONDFnN1 =\

zdemio.ContactBondListStrToNumpyArray(CONTACT, BOND)

tic = time.time()

allnum = cmd_bondplot(file,CurrentStep,BALLIdN1, BALLxyN2,BALLRadN1,\

WALLP1P2xyxyN4,ColorList,\

CONTACTId1Id2N2,BONDId1Id2N2,BONDFnN1,\

xmaxdefine, ymaxdefine, xmindefine, ymindefine,\

xmin,xmax,ymin,ymax,\

wbleft,wbright,wbbottom,wbtop,\

leftshow,rightshow,bottomshow,topshow,\

major_locator,minor_locator,fontsize,linewidth,pagesize)

toc = time.time()

logger.info('used {:.5}s'.format(toc-tic))

#allnum=[0,1,2,3,4]

return allnum

def cmd_bondplot(file,CurrentStep,BALLIdN1, BALLxyN2,BALLRadN1,

WALLP1P2xyxyN4,ColorList,

CONTACTId1Id2N2,BONDId1Id2N2,BONDFnN1,

xmaxdefine, ymaxdefine, xmindefine, ymindefine,

xmin,xmax,ymin,ymax,

wbleft,wbright,wbbottom,wbtop,

leftshow,rightshow,bottomshow,topshow,

major_locator,minor_locator,fontsize,linewidth,pagesize):

figbond=plt.figure(2)

axbond=plt.gca()

#	FnGEZero,FnLTZero = 0,0

FnGEZero,FnLTZero = zdemplot.plot_contactbond(figbond, axbond,

BALLIdN1, BALLxyN2,

CONTACTId1Id2N2,BONDId1Id2N2,BONDFnN1,

linewidth=linewidth)

logger.debug('wallshow')

if (wallshow == 'true'):

zdemplot.plot_wall(figbond,axbond,WALLP1P2xyxyN4,ColorList,linewidth=linewidth)

if (surfaceshow == 'true'):

zdemplot.plot_surface(figbond,axbond,BALLxyN2, BALLRadN1,linewidth=linewidth)

#set figure

zdemplot.zdem_fig_set(figbond,axbond,xmaxdefine, ymaxdefine, xmindefine, ymindefine,

xmin,xmax,ymin,ymax,

wbleft,wbright,wbbottom,wbtop,

leftshow,rightshow,bottomshow,topshow,

major_locator,minor_locator,fontsize,linewidth,pagesize)

VBOXfileJPG = file.replace('.dat','.jpg')

VBOXfileJPG = VBOXfileJPG.replace('all','bond')

figbond.savefig(VBOXfileJPG,dpi=dpi, bbox_inches="tight")#, pad_inches=0.0)

plt.close(figbond)

#print(file1[-10:])

# step contact_num bond_num pushnum(blue) pullnum(red)

return int(CurrentStep),CONTACTId1Id2N2.shape[0],BONDId1Id2N2.shape[0],FnGEZero,FnLTZero

allnum=[]

max_workers=min(max_workers,len(VBOXfile))

print("parallel num:",max_workers)

print("file num:",len(VBOXfile))

if __name__ == '__main__' :

with ProcessPoolExecutor(max_workers=max_workers) as executor:

VBOXfileList= VBOXfile

allnum=executor.map(read_and_gen_fig, VBOXfileList)

#test

#for file in VBOXfile:

#	allnum.append(read_and_gen_fig(file))

#print("allnum",allnum,flush=True)

#write contact and bond num to file

if bondplot =='true':

fileContactBondName='contactbond_num.txt'

allnum=list(allnum)

logger.info(allnum)

allnum=sorted(allnum,key=lambda x:x[0])

logger.info(allnum)

filecb = os.path.join(DataDir,fileContactBondName)

#print('filecb:',filecb)

with open(filecb,"w+") as f:

f.write("#{0:>15s} {1:>15s} {2:>15s} {3:>15s} {4:>15s} \n".format(\

"step","contact","bond","push(blue)","pull(red)"))

#f.write('# step contact_num bond_num pushnum(blue) pullnum(red)'+"\n")

for line in allnum:

#line = " ".join('%9s'%i for i in line)

#f.write(line+"\n")

f.write(" {0[0]:>15d} {0[1]:>15d} {0[2]:>15d} {0[3]:>15d} {0[4]:>15d}\n".format(line) )

// File: zdemio.py

#! /usr/bin/env python

# -*- coding: utf-8 -*-

import os, sys

#for pyinstall

from pathlib import Path

import numpy as np

def usage(softwareName):

#print("------do not give the dir of data , usage:----------")

print("用法: %s --dir=./data --dpi=300" %(softwareName) )

print("	-------参数说明----------")

print("	--dir   VBOX数据所在目录")

print("	--xmax 绘图X最大值")

print("	--ymax 绘图Y最大值")

print("	--xmin 绘图X最小值，默认0.0")

print("	--ymin 绘图Y最小值，默认0.0")

print("	--xmove 坐标沿x轴偏移量，默认0.0")

print("	--ymove 坐标沿y轴偏移量，默认0.0")

print("	--major_locator 主坐标刻度间隔，默认10000")

print("	--minor_locator 次坐标刻度间隔，默认1000")

print("	--fontsize 坐标刻度字体大小，默认9")

print("	--max_workers 并行进程数，默认24")

print("	--dpi 图片分辨率，默认600")

print("	--linewidth 线粗细，默认0.5")

print("	--pagesize 图片大小，单位cm，默认14")

print("	--leftshow 显示坐标轴左线框，取值true/false，默认true")

print("	--rightshow 显示坐标轴右边框，取值true/false，默认true")

print("	--bottomshow 显示坐标轴下线框，取值true/false，默认true")

print("	--topshow 显示坐标轴上边框，取值true/false，默认true")

print("	--wallshow 显示wall墙，取值true/false，默认true")

print("	--surfaceshow 显示变形边界true/false，默认true")

print("	--bondplot 绘制粘结连接关系true/false，默认false，黑色是没有粘结，红色是有粘结拉伸状态，蓝色是有粘结挤压状态")

print("	--ballplot 绘制颗粒true/false，默认true")

print("	--colormap 指定颜色配置文本文件，格式为10x3的矩阵，对应十个RGB值．")

print("")

print("	-v 打印版本号")

print("	-h 打印此帮助信息")

def get_file_list(DataDir,FileNamePrefix,FileNameSuffix):

"""获取指定目录下符合前后缀的文件列表。"""

pathDir = os.listdir(DataDir)

ListFile = [] #result

for FileName in pathDir:

#print ("s:%s"%(s))

newDir=os.path.join(DataDir,FileName)

#print ("newDir:%s"%(newDir))

if os.path.isfile(newDir) :         #Èç¹ûÊÇÎÄ¼þ

#print ("file:%s"%(newDir))

#if os.path.splitext(newDir)[1]==FileType:  #ÅÐ¶ÏÊÇ·ñÊÇtxt

#print ("txt:%s"%(newDir))

#tempFileDir  = os.path.split(newDir)[0] #

tempFileName = os.path.split(newDir)[1] #

#print ("tempFileName:%s"%(tempFileName))

#print (tempFileName.find('log')==-1)

#print ("newDir:%s"%(newDir))

#dont't save log file

if (tempFileName.find(FileNamePrefix) != -1) and  (tempFileName.find(FileNameSuffix) != -1):

#if tempFileDir.find('wu_zhenyun')==1:

#print ("don't have 'log '%s"%(newDir))

#backup_file(newDir,fixoldDir,backupDir)

#print ("tempFileName:%s"%(tempFileName[0:2]))

ListFile.append(newDir)

#print ("newDir:%s"%(newDir))

return ListFile

def read_data(filename):

"""

2019/01/07

LI ChangSheng @ Nanjing University

read zdem data

input：

[1] zdem data dir <./data>

output：

[1] wall, ball

example:

WALL,BALL,CONTACT,BOND,CurrentStep=read_data("./all_0000101000.dat")

"""

flag = 0

#header = []

WALL = []

BALL = []

CONTACT=[]

BOND=[]

GROUP = []

vbcontact= 'False'

vbbond='False'

zdemfile = open(filename, "r")#, encoding = 'utf-8')

for line in zdemfile:

#if "ball data" in line:

#	header.append(line)

#	flag = 1

#	continue

#print(line,'flag',flag)

if "     index         id      P1[0]" in line: #mark the beginning of wall data

flag = 1

continue

if "     index         id         xF" in line: #quit as the wall data goes to end

flag = 0

if "     index         id                    x" in line: #mark the beginning of cell data

flag = 2

continue

if "     index         id            m" in line: #quit as the cell data goes to end

flag = 5

continue

if "contact data ..." in line: #record contact

vbcontact='true'

continue

if "id1" in line and vbcontact=='true': #record contact

flag =3

continue

if "bond data ..." in line: #record contact

vbbond = 'true'

flag=4

vbcontact='false'

#print('12')

continue

if "       id1" in line and vbbond == 'true' : #record contact

flag =4

continue

if 'current_step' in line :

step_info = line.split()

CurrentStep=step_info[-1]

if flag == 0:

#header.append(line)

continue

if flag == 1:

wall_info = line.split()

WALL.append(wall_info)

if flag == 2:

#print line

ball_info = line.split()

#ball_info = [ float(i) for i in ball_info[2:6] ]

BALL.append(ball_info)

if flag == 3:

#print ("3")

contact_info = line.split()

contact_info = [ int(i) for i in contact_info[0:2] ]

CONTACT.append(contact_info)

if flag == 4:

#print line

bond_info = line.split()

bond_info = [ i for i in bond_info[0:3] ]

BOND.append(bond_info)

if flag == 5:

temp = line.split()

if len(temp) != 0:

GROUP.append([temp[1], temp[10]])

del WALL[-1] #del last NULL line

del BALL[-1] #del last NULL line

if CONTACT == []:

CONTACT =[1]

if BOND == []:

BOND =[1]

del CONTACT[-1] #del last NULL line

del BOND[-1] #del last NULL line

#print(CONTACT,BOND)

zdemfile.close()

#print(BOND)

return  WALL, BALL, CONTACT, BOND, CurrentStep, GROUP

def BallListStrToNumpyArray(BALL):

"""

转换为numpy数据类型matrix

输入参数：

[1]BALL  颗粒信息nx6 [ index id x y r color]

输出参数：

[1]BALLid     nx1

[2]BALLxyn2   nx2

[4]BALLr      nx1

[5]BALLcolor  nx1

"""

#list(map(int,BALL))

IDList    = [int(oneInfo[1])   for oneInfo in BALL]

UxList    = [float(oneInfo[2]) for oneInfo in BALL]

UyList    = [float(oneInfo[3]) for oneInfo in BALL]

RADList    = [float(oneInfo[4]) for oneInfo in BALL]

COLORList = [int(oneInfo[5])   for oneInfo in BALL]

BALLId=np.matrix(IDList).T

BALLx=np.matrix(UxList).T

BALLy=np.matrix(UyList).T

BALLxyN2 = np.hstack((BALLx,BALLy))

BALLRad=np.matrix(RADList).T

BALLColor=np.matrix(COLORList).T

return BALLId, BALLxyN2, BALLRad, BALLColor

def  WallListStrToNumpyArray(WALL):

"""

转换为numpy数据类型

输入参数：

[1]WALL  墙体信息nx6 [ index id P1x P1y P2x P2y ]

输出参数：

[1]WALLid           nx1 [id]

[2]WALLP1P2xyxyN4   nx4 [ P1x P1y P2x P2y ]

"""

IDList    = [int(oneInfo[1])   for oneInfo in WALL]

P1xList    = [float(oneInfo[2]) for oneInfo in WALL]

P1yList    = [float(oneInfo[3]) for oneInfo in WALL]

P2xList    = [float(oneInfo[4]) for oneInfo in WALL]

P2yList    = [float(oneInfo[5]) for oneInfo in WALL]

WALLId=np.matrix(IDList).T

WALLP1x=np.matrix(P1xList).T

WALLP1y=np.matrix(P1yList).T

WALLP2x=np.matrix(P2xList).T

WALLP2y=np.matrix(P2yList).T

WALLP1P2xyxyN4 = np.hstack((WALLP1x, WALLP1y, WALLP2x, WALLP2y))

# print('WALLP1P2xyxyN4', WALLP1P2xyxyN4)

return WALLId, WALLP1P2xyxyN4

def ContactBondListStrToNumpyArray(CONTACT, BOND):

"""

转换为numpy数据类型

输入参数：

[1]CONTACT  接触信息n x [ id1  id2  Fn  Fs   globalFx   globalFy ...]

[2]BOND     粘结信息n x [ id1  id2  Fn  Fs   globalFx   globalFy ...]

输出参数：

[1]CONTACTId1Id2N2    nx2 [id1 id2]

[2]BONDId1Id2N2       nx2 [id1 id2]

[3]BONDFnN1           nx1 [Fn]

"""

CONTACTId1List  = [int(oneInfo[0]) for oneInfo in CONTACT]

CONTACTId2List  = [int(oneInfo[1]) for oneInfo in CONTACT]

BONDId1List  = [int(oneInfo[0]) for oneInfo in BOND]

BONDId2List  = [int(oneInfo[1]) for oneInfo in BOND]

BONDFnList   = [float(oneInfo[2]) for oneInfo in BOND]

CONTACTId1=np.matrix(CONTACTId1List).T

CONTACTId2=np.matrix(CONTACTId2List).T

CONTACTId1Id2N2 = np.hstack((CONTACTId1, CONTACTId2))

BONDId1=np.matrix(BONDId1List).T

BONDId2=np.matrix(BONDId2List).T

BONDId1Id2N2 = np.hstack((BONDId1, BONDId2))

BONDFnN1=np.matrix(BONDFnList).T

return CONTACTId1Id2N2,BONDId1Id2N2,BONDFnN1

#def xy_move( WALLP1x, WALLP1y, WALLP2x, WALLP2y, BALLUx, BALLUy, xmove, ymove):

def xy_move( WALLP1P2xyxyN4, BALLxyN2, xmove, ymove):

"""

输入参数：

[1] WALLP1P2xyxyN4  墙体信息 nx4 [P1x P1y P2x P2y]

[1] BALLxyN2  颗粒信息 nx2 [x y]

[2] xmove 颗粒坐标x方向偏移量

[3] ymove 颗粒坐标y方向偏移量

"""

WALLP1P2xyxyN4 = WALLP1P2xyxyN4 + np.mat([[xmove,ymove,xmove,ymove]])

BALLxyN2 = BALLxyN2 + np.mat([[xmove,ymove]])

return WALLP1P2xyxyN4, BALLxyN2

def resource_path(relative_path):

"""

生成资源文件目录访问路径

说明： pyinstaller工具打包的可执行文件，运行时sys。frozen会被设置成True

因此可以通过sys.frozen的值区分是开发环境还是打包后的生成环境

打包后的生产环境，资源文件都放在sys._MEIPASS目录下

修改main.spec中的datas，

如datas=[('res', 'res')]，意思是当前目录下的res目录加入目标exe中，在运行时放在零时文件的根目录下，名称为res

"""

if getattr(sys, 'frozen', False):

base_path = sys._MEIPASS

else:

base_path = os.path.abspath(".")

return os.path.join(base_path, relative_path)

def GetColormapFile(colormap,VBOXscriptDir):

"""

获得 colormap 文件

输入参数：

[1] colormap  指定颜色配置文本文件路径，默认为'default'，格式为10x3的矩阵，对应十个RGB值．

输出参数：

[1] colormapfile  颜色配置文本文件路径

"""

if (colormap == 'default') :

colormapfile=VBOXscriptDir+'/res/ColorRicebal.txt'

my_file = Path(colormapfile)

if (not my_file.exists()):

#print("文件不存在，***colormapfile:",colormapfile)

# 指定的文件存在

colormapfile = resource_path(os.path.join("res","ColorRicebal.txt"))

else:

#默认取值见https://doc.geovbox.com/zdem/color/．建议直接制定该文件的绝对路径或者相对路径，

#如--colormap=/home/zhangsan/MyColorMap.txt或--colormap=./ＭyColorMap.txt．

#如果仅指定文件名，如--colormap=ＭyColorMap.txt，搜索顺序为 当前目录 > --dir指定的目录 > Home．")

colormapfile=colormap

#print("1:", colormapfile)

if ( os.path.isfile(colormapfile) == False ):

colormapfile=DataDir+colormap

if ( os.path.isfile(colormapfile) == False ):

colormapfile=os.path.join(os.environ['HOME'],colormap)

if ( os.path.isfile(colormapfile) == False ):

print("找不到", colormap)

#print("colormap:", colormapfile)

#with open(colormapfile) as f:

#    lines = f.readlines()

#    print(lines)

#    f.close()

return colormapfile

// File: zdemplot.py

# -*- coding: utf-8 -*-

"""

绘制颗粒及墙体

"""

import os

import shutil

import numpy as np

import time

import matplotlib.ticker as ticker

#from pylab import *

from math import ceil

import matplotlib.pyplot as plt

from matplotlib.collections import LineCollection

from matplotlib.collections import EllipseCollection

from matplotlib import colors as mcolors

from matplotlib.patches import Circle

from matplotlib.lines import Line2D

def get_color_map(filename):

"""获取颜色映射表"""

xfile = open(filename, "r")#, encoding = 'utf-8')

colorlist = []

for line in xfile:

ltmp = line.split()

#print (ltmp, '\n')

for i in range(len(ltmp)):

ltmp[i] = float(ltmp[i])

colorlist.append(ltmp)

xfile.close()

colormap={}

colormap['light gray']  = (colorlist[0],0)

colormap['green']       = (colorlist[1],1)

colormap['yellow']      = (colorlist[2],2)

colormap['red']         = (colorlist[3],3)

colormap['white']       = (colorlist[4],4)

colormap['black']       = (colorlist[5],5)

colormap['medium gray'] = (colorlist[6],6)

colormap['blue']        = (colorlist[7],7)

colormap['green blue']  = (colorlist[8],8)

colormap['violet']      = (colorlist[9],9)

return colorlist,colormap

def plot_ball(fig, ax, BALLxyN2, BALLRadN1, BALLColorN1, ColorList):

"""

输入参数：

[1] fig

[2] ax

[3] BALLxyN2   nx2

[4] BALLRadN1 nx1

[5] BALLColorN1 nx1

[6] ColorList

[0.85 0.85 0.85

0.00 1.00 0.00

1.00 1.00 0.00

1.00 0.00 0.00

0.90 0.90 0.90

0.15 0.15 0.15

0.50 0.50 0.50

0.00 0.00 1.00

0.00 1.00 1.00

1.00 0.00 1.00]

"""

ww = BALLRadN1 * 2.0

hh = ww

#print("ww.shape",ww.shape)

aa = np.zeros(ww.shape)

XY = BALLxyN2

# set color

#list(n)

#list[0]=(r,g,b,t)

# ...

#list[n]=(r,g,b,t)

facecolors = [ tuple(ColorList[c[0,0]]) for c in BALLColorN1 ]# c=[r,g,b,t]

circles = EllipseCollection(ww,hh,aa,

units='x',offsets=XY,

transOffset=ax.transData,

edgecolors='none',

facecolors=facecolors,

linestyles='solid')

ax.add_collection(circles)

def search_domain(WALLP1P2xyxyN4, BALLxyN2, BALLRad):

wallNum=WALLP1P2xyxyN4.shape[0]

ballNum=BALLxyN2.shape[0]

if (wallNum!=0):

Wleft,Wright,Wbottom,Wtop = search_domain_wall(WALLP1P2xyxyN4)

#print("w",Wleft,Wright,Wbottom,Wtop)

if (ballNum!=0):

Bleft,Bright,Bbottom,Btop = search_domain_ball(BALLxyN2, BALLRad)

#print("b",Bleft,Bright,Bbottom,Btop)

if (wallNum!=0):

if (ballNum!=0):

return min(Wleft,Bleft), max(Wright,Bright), min(Wbottom,Bbottom), max(Wtop,Btop)

else:

return Wleft, Wright,Bright, Wbottom, Wtop

else:

if (ballNum!=0):

return Bleft, Bright, Bbottom, Btop

else:

return 0.0,1.0,0.0,1.0

def search_domain_wall(WALLP1P2xyxyN4):

P1P2xyxymin = WALLP1P2xyxyN4.min(axis=0) # [P1xmin P1ymin P2xmin P2ymin]

P1P2xyxymax = WALLP1P2xyxyN4.max(axis=0) # [P1xmax P1ymax P2xmax P2ymax]

left   = min(P1P2xyxymin[0,0],P1P2xyxymin[0,2])

right  = max(P1P2xyxymax[0,0],P1P2xyxymax[0,2])

bottom = min(P1P2xyxymin[0,1],P1P2xyxymin[0,3])

top    = max(P1P2xyxymax[0,1],P1P2xyxymax[0,3])

return left,right,bottom,top

def search_domain_ball(BALLxyN2, BALLRad):

leftbottom = (BALLxyN2-BALLRad).min(axis=0)

righttop = (BALLxyN2+BALLRad).max(axis=0)

left = leftbottom[0,0]

bottom = leftbottom[0,1]

right = righttop[0,0]

top = righttop[0,1]

return left,right,bottom,top

def plot_wall(fig, ax, WALLP1P2xyxyN4, ColorList,linewidth=1):

WALLsegs=[np.row_stack([P1P2xyxy[0,0:2], P1P2xyxy[0,2:4]]) for P1P2xyxy in WALLP1P2xyxyN4 ]

#set color

#	colors = [mcolors.to_rgba(c)

#          for c in plt.rcParams['axes.prop_cycle'].by_key()['color']]

wallNum=WALLP1P2xyxyN4.shape[0]

#print("wallNum",wallNum)

WALLColors=np.zeros((wallNum,4))

WALLColors=WALLColors+np.array([0.0,0.0,0.0,1.0]) #black

#print("WALLColors",WALLColors)

WALLColorList=[tuple(c) for c in WALLColors ]

#facecolors = [ tuple(ColorList[c]) for c in BALLColor ]# c=[r,g,b,t]

WALLline_segments = LineCollection(WALLsegs, linewidths=linewidth, colors=WALLColorList, linestyles='solid')

ax.add_collection(WALLline_segments)

#	print("WALLColorList",WALLColorList)

def plot_surface(fig, ax, BALLxyN2, BALLRadN1, linewidth=1):

"""

输入参数：

[1] fig

[2] ax

[3] BALLxyN2   nx2

[4] BALLRadN1  nx1

"""

dx=200

dy=5000

left   = (BALLxyN2[:,0]-BALLRadN1).min()

right  = (BALLxyN2[:,0]+BALLRadN1).max()

nx= int(ceil((right - left)/dx))

ycol=np.zeros((2,nx))

#print("BALLUx.shape[0]:",BALLUx.shape[0])

for i in range(BALLxyN2.shape[0]):

#for Ux,Uy,rad in BALLUx, BALLUy, BALLRadN1:

Ux,Uy,rad = BALLxyN2[i,0], BALLxyN2[i,1], BALLRadN1[i]

index = int(ceil((Ux-left)/dx))

if(index>0):

index=index-1

if(Uy > ycol[0,index]):

ycol[0,index] = Uy+rad

ycol[1,index] = i #记住颗粒index

LineX=np.zeros(nx)

LineY=np.zeros(nx)

for i in range(nx):

LineX[i]=dx*i+left

if i>0 and i<(nx-1):

if abs(ycol[0,i] - ycol[0,i-1]) <dy and abs(ycol[0,i+1] - ycol[0,i])<dy:

LineY[i]=ycol[0,i]*0.5+ycol[0,i-1]*0.25+ycol[0,i+1]*0.25

else:

LineY[i]=ycol[0,i]

else:

LineY[i]=ycol[0,i]

ax.add_line(Line2D(LineX, LineY , linewidth=linewidth, color='black'))

def plot_contactbond(fig, ax, BALLIdN1, BALLxyN2,

CONTACTId1Id2N2,BONDId1Id2N2,BONDFnN1,

linewidth=0.2):

"""

将 BOND Id1Id2 映射到 BALL 索引，绘制 contact 线段和 bond 线段。

"""

BALLIdN1=BALLIdN1.T #列变成行

#	#plot contact

# 将[id1 id2]转换为[index1 index2]

CONTACTIDId1Id2index = Id1Id2ToIndex1Index2(BALLIdN1,CONTACTId1Id2N2)

CONTACTXYid1 = BALLxyN2[CONTACTIDId1Id2index[:,0],:] # nx2 [x,y]

CONTACTXYid2 = BALLxyN2[CONTACTIDId1Id2index[:,1],:]

CONTACTsegs=[np.vstack((id1xy, id2xy)) for id1xy,id2xy in zip(CONTACTXYid1,CONTACTXYid2) ]

#color

contactnum=CONTACTId1Id2N2.shape[0]

CONTACTColors=np.zeros((contactnum,4))

CONTACTColors=CONTACTColors+np.array([0.0,0.0,0.0,1.0]) #black

CONTACTColorList=[tuple(c) for c in CONTACTColors ]

CONTACTline_segments = LineCollection(CONTACTsegs, linewidths=linewidth, colors=CONTACTColorList, linestyles='solid')

ax.add_collection(CONTACTline_segments)

#plot bond

bondnum=BONDFnN1.shape[0]

# 将[id1 id2]转换为[index1 index2]

BONDId1Id2index = Id1Id2ToIndex1Index2(BALLIdN1,BONDId1Id2N2)

BONDXYid1 = BALLxyN2[BONDId1Id2index[:,0],:] # nx2  [x, y]

BONDXYid2 = BALLxyN2[BONDId1Id2index[:,1],:] # nx2  [x, y]

segs=[np.vstack((id1xy, id2xy)) for id1xy,id2xy in zip(BONDXYid1,BONDXYid2) ]

#color

FnLTZero=0 #Fn<0 red

FnGEZero=0 #Fn>=0 blue

#	colors = [mcolors.to_rgba(c)

#          for c in plt.rcParams['axes.prop_cycle'].by_key()['color']]

indexRed=np.where(BONDFnN1<0)

indexBlue=np.where(BONDFnN1>=0)

FnLTZero=indexRed[0].shape[0] #Fn<0 red

FnGEZero=indexBlue[0].shape[0] #Fn>=0 blue

Colors=np.zeros((bondnum,4))

Colors[indexRed,:]=np.array([1.0,0.0,0.0,1.0]) #red

Colors[indexBlue,:]=np.array([0.0,0.0,1.0,1.0]) #blue

ColorList=[tuple(c) for c in Colors ]

line_segments = LineCollection(segs, linewidths=linewidth, colors=ColorList, linestyles='solid')

ax.add_collection(line_segments)

return FnGEZero,FnLTZero

def Id1Id2ToIndex1Index2(BALLIdN1,Id1Id2N2):

"""将 [id1 id2] 转换为 [index1 index2]，逐行查找避免广播大矩阵内存溢出。"""

bondnum=Id1Id2N2.shape[0]

Id1Id2indexN2=np.mat(np.zeros_like(Id1Id2N2,dtype=int)) # nx2

for i in np.arange(bondnum):

id1id2=Id1Id2N2[i,:].T #取出一个bond， [id1, id2]

index1index2 = np.where(BALLIdN1==id1id2)[-1]

Id1Id2indexN2[i,:]=index1index2 #[index1 index2]

return Id1Id2indexN2

def fig_set(fig):

"""

输入参数：

[1] fig

"""

#plt.plot(VBOXx,VBOXy,'.')

plt.xlim(left=0.0)#, xmax = 0.16)

#plt.xlim(right = 120)

plt.ylim(bottom=0.0)

plt.axis('equal')   #changes limits of x or y axis so that equal increments of x and y have the same length

#plt.ylim(top =20)

#plt.show()

wi,hi=fig.get_size_inches()

#print("wi", wi, "hi",hi)

wcm=8 #cm

#hcm=

winch=wcm/2.54

hinch=winch/wi*hi

#print("winch", winch, "hinch",hinch)

fig.set_size_inches(w=winch,h=hinch)

def zdem_fig_set(fig,ax,xmaxdefine, ymaxdefine, xmindefine, ymindefine,

xmin,xmax,ymin,ymax,

wbleft,wbright,wbbottom,wbtop,

leftshow,rightshow,bottomshow,topshow,

major_locator,minor_locator,fontsize,linewidth,pagesize):

if (xmaxdefine=='true'):

right  = xmax

else:

right=min(wbright,xmax)

if (ymaxdefine=='true'):

top  = ymax

else:

top  =min(wbtop,ymax)

if (xmindefine=='true'):

left  = xmin

else:

left=max(wbleft,xmin);

if (ymindefine=='true'):

bottom  = ymin

else:

bottom=max(wbbottom,ymin);

#print(left,right,bottom,top)

realticks=range(0, 400000, int(major_locator) )

showticks= [str(x) for x in range(0, 400, int(int(major_locator)/1000) )]

plt.xticks(realticks,showticks)

plt.yticks(realticks,showticks)

ax.xaxis.set_minor_locator(ticker.MultipleLocator(minor_locator))

ax.yaxis.set_minor_locator(ticker.MultipleLocator(minor_locator))

# 设置刻度字体大小

plt.xticks(fontsize=fontsize)

plt.yticks(fontsize=fontsize)

plt.tick_params(labelsize=fontsize)

###设置坐标轴的粗细

ax.spines['bottom'].set_linewidth(linewidth);###设置底部坐标轴的粗细

ax.spines['left'].set_linewidth(linewidth);####设置左边坐标轴的粗细

ax.spines['right'].set_linewidth(linewidth);###设置右边坐标轴的粗细

ax.spines['top'].set_linewidth(linewidth);####设置上部坐标轴的粗细

#set 刻度线粗细 刻度线与标签的间隔

ax.tick_params(which='both',width=linewidth, pad=1);

#wi,hi=fig.get_size_inches()

wi=right-left

hi=top-bottom

#	print("wi", wi, "hi",hi)

#	hi=3.0

wcm=pagesize #cm

#hcm=

winch=wcm/2.54

hinch=winch/wi*hi

#	hinch=1

#print("winch", winch, "hinch",hinch)

fig.set_size_inches(w=winch,h=hinch)

#plt.axis('equal')   #changes limits of x or y axis so that equal increments of x and y have the same length

#	plt.plot(VBOXx,VBOXy,'.')

#print(left,right,bottom,top)

plt.xlim(left=left)#, xmax = 0.16)

plt.xlim(right=right)

plt.ylim(bottom=bottom)

plt.ylim(top=top)

#去掉边框

#print("topshow:",topshow)

if (topshow=='false'): ax.spines['top'].set_visible(False)

if (rightshow=='false'): ax.spines['right'].set_visible(False)

if (bottomshow=='false'): ax.spines['bottom'].set_visible(False)

if (leftshow=='false'): ax.spines['left'].set_visible(False)

// File: setup.py

# -*- coding: utf-8 -*-

import setuptools

with open("README.md", "r", encoding="utf-8") as fh:

long_description = fh.read()

setuptools.setup(

name="zdem-area-conservation",

version="1.0.0",

description="ZDEM 颗粒分布与面积守恒分析工具 / Area conservation analysis for ZDEM granular simulations",

long_description=long_description,

long_description_content_type="text/markdown",

url="https://github.com/Phoenix0531-sudo/ZDEM_Area_Conservation",

packages=setuptools.find_packages(),

install_requires=[

"numpy>=1.21.0",

"scipy>=1.7.0",

"matplotlib>=3.4.0",

],

python_requires=">=3.8",

classifiers=[

"Development Status :: 4 - Beta",

"Intended Audience :: Science/Research",

"License :: OSI Approved :: MIT License",

"Natural Language :: Chinese (Simplified)",

"Natural Language :: English",

"Operating System :: OS Independent",

"Programming Language :: Python :: 3",

"Programming Language :: Python :: 3.8",

"Programming Language :: Python :: 3.9",

"Programming Language :: Python :: 3.10",

"Programming Language :: Python :: 3.11",

"Topic :: Scientific/Engineering",

"Topic :: Scientific/Engineering :: Physics",

],

)

// File: zdem_area_conservation/__init__.py

# -*- coding: utf-8 -*-

"""ZDEM 颗粒分布与面积守恒分析包"""

__version__ = '1.0.0'

__license__ = 'MIT'